After 1959-1964 refit  :
Displacement: 43,000 tons standard ; 50,536 tons full load
Dimensions: 813.5 oa x 171 x 34.5 feet
Range: 7,000 nmiles at 14 knots ; 3,200 tons fuel oil
Aviation Facilities: 409 x 67 x 17.5 ft (upper) & 172 x 54 x 17.5 ft (lower) hangars ; 800 x 160 ft 8-degree angled flight deck ; 4 35,000 lb arrester wires ; 2 BS5 50,000 lb steam catapults, 1 port bow and 1 waist ; 2 40,000 lb lifts (35 second cycle), 54ft long x 44ft wide fwd., 54 x 33 feet aft. ; 487,480 gallons aviation fuel (AVCAT)
Aircraft: 44
Armament: 4 twin 4.5-inch / 45 Mk 6 DP turrets ; 6 GWS22 Sea Cat point-defence SAM quad launchers ; 4 single 3 pdr saluting guns
Radars: 3D Fighter Control - Type 984 ; Air Search - Type 965 ; Carrier Approach - Type 963

Recommissioned at Devonport 14 May 1964.
Repairs at Devonport 24 May to August 1965.
Returned to Devonport for refit 28 August 1966.